E2. Quartz Crystal Oscillator (Simulation) (10.0 points)

https://taisoncar.github.io/quartz/

Credit: Russian Team Selection Test (X) 2026 [https://pho.rs/p/4788]

A quartz crystal oscillator is a type of electrical circuit that utilizes the piezoelectric properties of quartz crystal to generate electrical oscillations. Quartz crystal oscillator is widely used to keep track of time in watches and computers due to its stability and accuracy.

To analyze the behavior of a quartz crystal in electrical circuits, we can model it as an equivalent circuit shown in the figure below.

In this problem, we will investigate the parameters of a quartz crystal through a program that simulates the following circuit:

The resonant frequency of the quartz crystal used in this simulation is known to be close to f0=2 MHz. The circuit is powered by an AC source U=U0cos⁡(ωt) with angular frequency ω=2πf. The voltage amplitude of the AC source U0 can be assumed to be constant at 500 mV. A resistor R0=5.1 kΩ is connected in series to the quartz crystal.

Note: Refer to the end of this paper for the instructions of the simulation program.

Part A. Measurements without external capacitor (5.7 points)

For this part, avoid connecting any external capacitor by inputting Cext=0.

| A1. | Determine the frequency fmax at which the voltage amplitude U2 across the quartz crystal is maximized. | 0.2 pt |
| --- | --- | --- |

fmax=2000040 Hz

| A2. | Determine the frequency fmin at which the voltage amplitude U2 across the quartz crystal is minimized. | 0.1 pt |
| --- | --- | --- |

fmax=1999931 Hz

| A3. | Measure the voltage amplitude U2 across the quartz crystal and the phase difference φ between the signals on the two oscilloscope channels as frequency f changes. | 2.8 pt |
| --- | --- | --- |

*generate csv through python program*

| A4. | Express the magnitude of the impedance of the quartz crystal Z in terms of R0, U2, U0 and φ. | 0.5 pt |
| --- | --- | --- |

U2eiωt+φU0eiωt=ZR0+Z

U2U0eiφ=ZR0+Z

Z=R0eiφU0U2-eiφ

Z=R0eiφU0U2-eiφ

Z=R0U0U2-cosφ2+sin2φ

| A5. | Plot the magnitude of the impedance of the quartz crystal ¦Z¦ against frequency f. | 2.0 pt |
| --- | --- | --- |

| A6. | From the plotted graph, find the resonant frequency fres and anti-resonant frequency fanti at which ¦Z¦ reaches its maximum and minimum values respectively. | 0.1 pt |
| --- | --- | --- |

fres=2000032 Hz

fanti=1999933 Hz

Part B. Deriving useful expressions (0.6 points)

The impedance of a quartz crystal can be written in the form

Zω=ωs2-ω2+iωαiωC0(ωp2-ω2+iωα)

where ωs,ωp and α do not depend on ω.

| B1. | Express ωs,ωp and α in terms of R,L,C and C0. | 0.3 pt |
| --- | --- | --- |

1Z=11iωC+iωL+R+iωC0

1Z=1+iωC01iωC+iωL+R1iωC+iωL+R

Z=1LC-ω2+iωRLiωL+iωC01LC-ω2+iωRL

Z=1LC-ω2+iωRLiωC01LC0+1LC-ω2+iωRL

By comparison:

ωs=1LC;   ωp=1LC0+1LC;   α=RL

| B2. | Express Z2 in terms of ω,ωs,ωp,α and C0. | 0.1 pt |
| --- | --- | --- |

Z2=ωs2-ω2+iωα2ωC0iωp2-ω2-ωα2

Z2=ωs2-ω22+ωα2ωC02 ωp2-ω22+ωα2

From here onwards, assume the following approximations hold for the range of frequencies we are working on in this problem:

Zω≈ωs2-ω2+iωpαiωpC0(ωp2-ω2+iωpα)

α≪ωp-ωs;   α≪ωp;   α≪ωs

φfanti=φfres=0

| B3. | Show that ωp=2πfres and ωs=2πfanti . | 0.2 pt |
| --- | --- | --- |

ω=2πfres , ω=2πfanti whendZdω=0

Let Z2ω=f(ω)g(ω)

dZdω=12|Z|f'g-fg'g2

Since1Zg2≠0 for all ω,

dZdω=0 when  f'g-fg'=0

Z2≈ωs2-ω22ωpC02ωp2-ω22

-2ω2ωs2-ω2ωpC02 ωp2-ω22-ωpC02ωs2-ω22-2ω2ωp2-ω2=0

ωs2-ω2ωp2-ω2ωp2-ωs2=0

Since ωp≠ωs, ω=ωs,ω=ωp

Plugging the two frequencies into Z2:

Z2ωp>Z2ωs

Thus, ωp=2πfres and ωs=2πfanti

Alternative:

-2ω2ωs2-ω2ωpC02 ωp2-ω22+ωpα2-ωpC02ωs2-ω22+ωpα2-2ω2ωp2-ω2=0

ωs2-ω2ωp2-ω22+ωpα2-ωp2-ω2ωs2-ω22+ωpα2=0

ωsωp2-ω22+ωsωpα2-ωωp2-ω22-ωωpα2-ωpωs2-ω22-ωp4α2+ωωs2-ω22+ωωpα2=0

ωsωp2-ωsω2-ωpωs2+ωpω2ωsωp2-ωsω2+ωpωs2-ωpω2+ωpα2ωs2-ωp2+ω2ωs2-ωp2ωs2+ωp2-2ω2=0

ωp-ωsωp+ωsωsωp+ω2ωsωp-ω2+ωs2-ωp2ωpα2+ω2ωs2+ωp2-2ω2=0

ωs2-ωp2ω4-ωsωp2+ωpα2+ω2ωs2+ωp2-2ω2=0

ωs2-ωp2ωpα2+ω2ωs2+ω2ωp2-ωs2ωp2-ω4=0

ωs2-ωp2ωpα2+ωs2(ω2-ωp2)+ω2(ωp2-ω2)=0

ωs2-ωp2ωpα2+(ω2-ωp2)(ωs2-ω2)=0

Since ωs≠ωp,

ωpα2+ω2-ωp2ωs2-ω2=0

-ω4+ωs2+ωp2ω2+ωpα2=0

ω=ωs2+ωp2±ωs2+ωp22+4ωp2(α2-ωs2)2

Using the approximation α≪ωs,

ω=ωs,ω=ωp

Plugging the two frequencies into Z2:

Z2ωp>Z2ωs

Thus, ωp=2πfres and ωs=2πfanti

Part C. Measurements with external capacitor (1.2 points)

When an external capacitor Cext is connected in parallel to the quartz crystal, the resonant frequency fres shifts while the anti-resonant frequency fanti remains constant.

| C1. | Measure fres and fanti as Cext changes. | 0.4 pt |
| --- | --- | --- |

| fres-f0 (Hz) | fanti-f0(Hz) | X=1fres2/fanti2-1 | Cext (pF) |
| --- | --- | --- | --- |
| 32 | -67 | 10100.42 | 0 |
| 17 | -67 | 11904.11 | 10 |
| 7 | -67 | 13512.81 | 20 |
| -1 | -67 | 15150.76 | 30 |
| -8 | -67 | 16948.33 | 40 |
| -13 | -67 | 18517.65 | 50 |
| -17 | -67 | 19999.08 | 60 |
| -21 | -67 | 21738.15 | 70 |
| -23 | -67 | 22726.26 | 80 |
| -26 | -67 | 24389.18 | 90 |
| -28 | -67 | 25639.92 | 100 |

| C2. | Plot a linear graph using the measured data. | 0.8 pt |
| --- | --- | --- |

When an external capacitor Cext is connected, we only need to make a simple change to the expression of ωp using the concept of equivalent capacitance:

C0→C0+Cext

ωpCext=1L(C0+Cext)+1LC

LCωp2=CC0+Cext+1

Cext=CLCωp2-1-C0=C1ωp2/ωs2-1 -C0=C1fres2/fanti2-1 -C0

Let X=1fres2/fanti2-1

A linear graph of Cext against X is plotted:

Part D. Determining the quartz crystal’s parameters (2.5 points)

| D1. | Determine the values of L, C and C0. | 1.3 pt |
| --- | --- | --- |

From the graph in C2,

C=0.0064 pF=6.4 fF

C0=66.7 pF

2πfanti=1LC

L=14π2fanti2C=0.99 H

| D2. | Determine the value of R. | 1.2 pt |
| --- | --- | --- |

Using the result from B2,

Z2=ωs2-ω22+ωα2ωC02 ωp2-ω22+ωα2

Using Cext=0, ω=ωs and ω=ωp,

Zanti2=ωsα2ωsC02 ωp2-ωs22+ωsα2

α=ωp2-ωs21ZantiC02-ωs2=2π(fres2-fanti2)fanti12πfantiZantiC02-1 =211.473

Zres2=ωs2-ωp22+ωpα2ωpC02 ωpα2

α=ωp2-ωs2ωpZresωpCo2-1=2π(fres2-fanti2)fres2πfresZresCo2-1=136.246

R=Lαavg=172 Ω

Instructions for the simulation program

The program simulates the voltage measurements of an oscilloscope with two channels CH1 and CH2 connected to the circuit as shown in the diagram on the first page.

The oscilloscope measures voltage as the electric potential at the point of connection with respect to the ground (GND).

The program takes two inputs within given range:

| Input | Description | Valid Range |
| --- | --- | --- |
| Cext | The capacitance in pF of the capacitor connected in parallel to the quartz crystal Note: input zero for part A to represent no capacitor connected in parallel | 0 to 100 pF |
| ∆f | The difference between the frequency f and f0=2 MHz, ∆f=f-f0 in Hz | -200 to +200 Hz |

After some calculations, the program will display two outputs that simulate the readings from an oscilloscope:

| Output | Description |
| --- | --- |
| U2 | The amplitude of the voltage reading on CH2 in mV |
| ∆t | The time delay in nanoseconds (ns) between the two closest peaks of the waveforms on CH1 and CH2, with positive values representing CH2 leads CH1 and vice versa |